# METHOD OF STATEMENT

Date: 10th Sep 2018

Description of work

To hoist up 6 unit packages to 3rd floor through door opening by using platform and mobile/lorry crane

Machine Spec

Package 1: 420 x 230 x 220 cm   /   Wt: 3, 000 kgs

Package 2: 420 x 230 x 220 cm   /   Wt: 3, 150 kgs

Package 3: 350 x 230 x 140 cm   /   Wt: 2, 650 kgs

Package 4: 450 x 230 x 180 cm   /   Wt: 3, 200 kgs

Package 5: 300 x 230 x 140 cm   /   Wt: 1, 100 kgs

Package 6: 440 x 230 x 140 cm   /   Wt: 1, 800 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

11/09/2018    -    10:00 am

Location of Operation

#### 3 Changi North St 2

Speed Mark Logistics Pte Ltd

Equipment Use

1 x 50 ton mobile/lorry crane

1 x 35 ton lorry crane

1 x low loader

1 x platform

Wire/webbing sling/shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoe, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Mobile/lorry crane to park at designated hoisting area for the lift

- Hoist up platform to 3rd floor door opening

- Stabilize platform effectively and ensure it is fully rested on the opening slab

- Tied back platform to a competent point and secured using 35 ton lorry crane

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- 50 ton mobile crane to hoist up machine to 3rd floor

- Lower down machine slowly onto platform

- Insert rollers/pallet truck to machine base

- Shift out machine from platform and position into customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175