# METHOD OF STATEMENT

Date: 24th August 2018

Description of work

To unload 1 unit machine from M’sia low loader, shift and position into customer premise at ground floor

Machine Spec

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

24/08/2018   –   10:00am

Location of Operation

#### No 3 Joo Koon Circle

AME International Pte Ltd

Equipment Use

1 x 50 ton mobile crane

Lorry crane

Heavy duty rollers

Wire/webbing sling

Forklift

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Setting up of mobile crane at designated location

- Hoist down machine from M;sia low loader to near ground level

- Insert heavy duty rollers to machine base

- Transport machine into production area

- Position machine at the designated location

- Jack up machine, pull out rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris, if any will be clear and dispose

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175