# METHOD OF STATEMENT

Date: 29th May 2017

Description of work

To hoist down 2 unit machine from 2nd floor by using platform and mobile/lorry crane, load onto lorry chassis and deliver warehouse for packing

Machine Spec

2 x MILLTAP 700 DMG Compact Machining Center

Dim: 300 x 230 x 295 cm

Wt: 5, 000 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

03/06/17   -   10:00 am

Location of Operation

#### 3 Joo Koon Circle – 2nd floor

AME

Equipment Use

1 x 50 ton mobile crane

1 x 50 ton lorry crane

1 x platform

Wire sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris, will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry/mobile crane operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                        All our workers are fully equip with standard P.P.E. (safety shoes, helmet, safety harness etc..) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Mobile/lorry crane to park at designated hoisting area for the lift

- Hoist up platform to 2nd level opening

- Stabilize platform effectively and ensure it is fully rested on the opening slab

- Tied back platform to a competent point and secured using 50 ton lorry crane

- Use strong belt to secure front left and right of platform to prevent platform swing outward

- Shift out machine from production and load onto platform

- 50 ton mobile crane to hoist down machine and load onto lorry chassis

- Secured machine with strapping belt prior to moving out from premise and deliver to warehouse for packing

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris, if any will be clear and dispose

- Job complete

Remark

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175