# METHOD OF STATEMENT

Date: 13th Oct 2020

Description of Work

To unload 12 packages of 1 unit machine from 1 x 40ft OT and 1 x 40ft HQ container, shift and position into premise at ground floor

Machine Spec

Laser Cutting Machine / Bysmart Fiber 3015

1 x 40ft HQ OT container

P 1: 510 x 225 x 240 cm / Wt: 4, 100 kg

P 2: 272 x 195 x 216 cm / Wt: 2, 155 kg

P 3: 300 x 170 x 150 cm / Wt: 1, 200 kg

P 4: 74 x 58 x 60 cm / Wt: 50 kg

1 x 40ft OT container

P 1: 431 x 217 x 120 cm / Wt: 2, 587 kg

P 2: 324 x 103 x 140 cm / Wt: 536 kg

P 3: 324 x 75 x 103 cm / Wt: 483 kg

P 4: 324 x 75 x 90 cm / Wt: 400 kg

P 5: 262 x 180 x 110 cm / Wt: 800 kg

P 6: 120 x 85 x 80 cm / Wt: 245 kg

P 7: 120 x 85 x 80 cm / Wt: 435 kg

P 8: 220 x 110 x 50 cm / Wt: 150 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

Est: 20/10/2020

Time of Operation

09:45 am

Location of Operation

#### 36 Joo Koon Circle

Trend Technologies (S) Pte Ltd

Equipment Use

1-2 x 633 lorry loader

1 x forklift

Rollers/pallet truck

Wire/webbing sling

Crowbar/hand jack

Obstacles

No obstacles on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry crane operator and forklift operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated location for the lift

- Open up container canvas

- Secured sling to machine hoisting point

- Hoist up machine from container and load onto the ground

- Uncrate case

- Hoist up and detach machine from base

- Swing crane away from base

- Lower down machine to near ground level

- Insert rollers/pallet truck under machine base

- Shift and transport machine into factory premise

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175