# METHOD OF STATEMENT

Date: 11th Sep 2019

Description of Work

To unload 1 unit machine and parts from low loader, assist engineer to install machine parts, after install, shift and position into production at ground floor

Machine Spec

Femco BMC-110R CNC Horizontal Boring Mill – To assemble outside factory

Total weight: 25, 750 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

13/09/2019 -   To deliver machine accessories and small parts

16/09/2019 -   To deliver main body and column

Location of Operation

#### 37 Joo Koon Rd

#### Energy Pipeline Services Pte Ltd

Equipment Use

1 x 50 ton mobile crane

1-2 x 633 lorry loader

1 set wire sling

1 x forklift

Shackle

1 set heavy duty rollers

1 x low loader

Obstacles

To clear all obstacles on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy mobile crane at designated hoisting location for the lift

- Reverse low loader to near crane

- Hoist up machine main body from low loader and unload to the ground

- Hoist up machine parts

- Assist engineer to install machine parts onto main body

- After installing, to shift and transport machine into inside factory

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175