# METHOD OF STATEMENT

Date: 24th Oct 2018

Description of work

To shift out 1 unit machine and accessories from production room to loading area, hoist up and load onto 1 x 20ft FR container. To secured machine with lashing wire and choking wood

Machine Spec

1 unit HPX-2520

Dim: 5000 x 4087 x 3630 mm

Wt: 14, 000 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

30/10/2018   -   10:00 am

Location of Operation

#### No 38M Penjuru Rd

#### SHIN GUAN Pte Ltd

Equipment Use

1 x 50 ton mobile crane

1 x 50 ton lorry crane

1 set rollers/pallet truck

1 set wire/webbing sling/shackle

1 x Spreader Bar

Plastic roll, canvas, shrink wrap (to cover machine after being load onto container)

Obstacles

No obstacle on loading and crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Jack up machine to 4” high

- Insert rollers to machine base

- Shift out machine from production to hoisting bay

- 50 ton mobile crane to hoist up machine and load onto FR container

- Release all sling from machine hoisting point

- To cover machine with plastic roll, canvas and shrink wrap

- Secured machine to container with lashing belt and choking wood

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175