# METHOD OF STATEMENT

Date: 03rd Sep 2018

Description of work

1) To unload 1 unit machine from 1 x 20ft GP container, hoist up to 2nd floor by using platform and mobile/lorry crane

2) To shift out and hoist down 1 unit CNC Lathe from 2nd floor and load onto forwarder lorry

Machine Spec

GLS-150 CNC Lathe

Dim: 210 x 150 x 170 cm

Wt: 3, 500 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

Est: 14/09/2018   -   10:00 am

Location of Operation

#### 39 Joo Koon Circle

Nutek Pte Ltd

Equipment Use

1 x 50 ton mobile crane

1 x 50 ton lorry crane

Lorry crane

1 x platform

Wire Sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Assignment of traffic controller to control movement within the vicinity                                                            All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervised by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Mobile/lorry crane to park at designated hoisting area for the lift

- Hoist up platform to 2nd floor opening

- Tied back platform to a competent point and secured using 50 ton lorry crane

- Use strong belt to secured front left and right of platform to prevent platform swing outward

- 50 ton mobile crane to hoist up machine to 2nd floor

- Lower down machine and load onto platform

- Insert rollers/pallet truck to machine base

- Shift out from platform and position into customer production room

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175