# METHOD OF STATEMENT

Date: 24th July 2017

Description of work

To hoist up 1 unit Furnace to 3rd floor by using platform and mobile crane

Furnace Spec

Dim: 5.7m x 1.7m x 1.75m

Wt: 3, 600 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

26/07/2017   -   After 12:00 pm

Location of Operation

#### No 2 Tuas Link 4 - 3rd Floor

#### 3M Singapore

Equipment Use

1 x 50 ton mobile crane

1 x lorry crane

Rollers/pallet truck

1 x platform

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Mobile crane to park at designated location for the hoist

- Hoist and load furnace into platform

- Strap and secured furnace onto platform with strapping belt

- Hoist up platform together with furnace to 3rd floor opening

- Stabilize platform effectively and ensure it is fully rested on the opening slab

- Use strong belt to secure front left and right of platform to prevent platform swing outward

- Insert rollers/pallet truck to furnace base

- Shift out furnace from platform and position into customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris, if any will be dispose and clear

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175