# METHOD OF STATEMENT

Date: 03rd Sep 2019

Description of work

To hoist up and load machine cases onto 6 x 40ft HQ, 1 x 40ft FR and 1 x 40ft OT container, secured with chocking wood and lashing wire

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

03-04/09/2019

Time of Operation

09:45 am

Location of Operation

#### No 4 Gul Way

National Oilwell Varco Pte Ltd

Equipment Use

1 x 633 lorry loader

1 x forklift

Rollers/pallet truck

Wire sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated hoisting location

- Insert sling under machine case

- Hoist up case onto container

- Secured machine with lashing belt and choking wood

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175