# METHOD OF STATEMENT

Date: 19th July 2023

Description of Work

To shift out 1 unit machine from production to loading area, using forklift to fork up onto lorry chassis

Machine Spec

1 unit Goodway GA2000

Dim: 270 x 163 x 173 cm

Wt: 3, 500 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

24/07/23

Time of Operation

Afternoon schedule

Location of Operation

#### 5 Bedok South Rd

#### Yokogawa Engineering Asia Pte Ltd

Equipment Use

2 x lorry loader

1 x 4.5 ton forklift

Rollers/pallet truck

Wire/webbing sling

Canvas, metal sheet (to lay out for floor protection)

Obstacles

No obstacles on loading/unloading area

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman & lorry loader operator will be involve in this operation

Safety Aspect

Barricade operation area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine & lifting gears with M.O.M certificate

# Job scope

- Jack up machine to 3” high using hand jack

- Insert rollers/pallet truck to machine bed

- To shift out machine from production to loading area

- Using 4.5ton forklift to fork up machine

- Load up machine onto lorry chassis

- Secured machine with strapping belt prior to moving out of customer premise

- Transport and deliver machine to Eric Wong warehouse

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani / Kevin Wong

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175