# METHOD OF STATEMENT

Date: 02nd Aug 2023

Description of work

To shift out 1 equipment from customer premise to loading area, hoist up onto lorry chassis and deliver to scrapyard for disposal

Equipment Spec

1 unit FW2000M Anilux Roll Cleaner

Dim: 230 x 55 x 130 cm

Wt: 900 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

08/08/23 (Tue)

Time of Operation

09:45 am

Location of Operation

#### 5 Cleantech Loop,

#### Cleantech 2, Blk B, CT2B

Equipment Use

1 x lorry crane

Pallet truck

Canvas/PVC (to lay out for floor protection)

Webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated loading area

- Shift out equipment from customer premise

- Transport equipment to loading area

- Insert sling to crane hook and secured to rigging point of equipment

- Lorry loader to hoist up and load equipment onto lorry chassis

- Release sling from rigging point of machine

- Secured with strapping belt prior to moving out of premise

- Transport and deliver to scrapyard for disposal

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175