# METHOD OF STATEMENT

Date: 28th Mar 2018

Description of work

To hoist up 5 unit machine to 2nd level of building by using lorry crane

Machine Spec

3 unit SHCM 97VS Milling

Dim: 179 x 185 x 206 cm

Wt: 2, 078 kgs each

2 unit Harrison M300 Conventional Lathe

Dim: 1.68m x 1.02m x 1.40m

Wt: 710 kg each

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

29/03/2018

Time Of Operation

09:45 am

Location of Operation

#### 50 Nanyang Ave

Nanyang Technological University

Equipment Use

1 x lorry crane

Wire/webbing sling

6/8.5 ton shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, safety harness etc..) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lorry crane to park at designated hoisting area for the lift

- Insert webbing sling onto machine hoisting point and hook onto crane

- Hoist up machine slowly to 2nd floor opening

- Lower down machine to near ground level

- Insert pallet truck to machine base

- Shift and position machine to the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175