# METHOD OF STATEMENT

Date: 12th Feb 2018

Description of Work

To unload 1 unit machine from lorry chassis, uncrate, shift and position into customer premise at ground floor

Machine Spec

1 unit V500G Wire Cut

Dim: 2215 x 2480 x 2205 mm

Wt: 3, 200 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

22/02/2018   -   10:00 am

Location of Operation

#### 50 Nanyang Ave HW-3

Nanyang Technological University

Equipment Use

1 x lorry crane

Pallet truck

Wire/webbing sling

Obstacles

No obstacles on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, etc..)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lorry crane to park at designated hoisting area

- Hoist down machine case from lorry chassis

- Uncrate machine case

- Hoist and detach machine from base

- Swing crane to unloading bay

- Lower down machine to near ground level

- Insert pallet truck to machine base for the move

- Transport machine into premise at ground floor

- Position machine to the designated location

- Unload machine from pallet truck and let it sit on machine plate

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175