# METHOD OF STATEMENT

Date: 02nd Mar 2020

Description of work

To hoist down 2 unit machine from lorry chassis, load onto platform, shift and position into premise at ground

Machine Spec

1 unit Mazak QT200 SG

Dim: 2630 x 1680 x 1703 mm

Wt: 4, 500 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

06/03/2020

Time of Operation

12:30 pm

Location of Operation

#### 500 Dover Rd – Singapore Polytechnic

W1313, School of Mechanical & Aeronautical Engineering

Equipment Use

2 x 633 PALFINGER lorry loader

1 x platform - to lay out for machine pathway

1 set rollers/pallet truck

Wire/webbing sling

Canvas/metal plate (for epoxy floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated location

- Hoist down and lay out platform for machine pathway

- Hoist up machine from lorry chassis and load onto platform

- Insert pallet truck/rollers onto machine base

- Shift out machine from platform and

- Transport machine into premise at ground floor

- Position machine at the designated location

- Unload machine from pallet truck/roller and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175