# METHOD OF STATEMENT

Date: 14th Feb 2023

Description of work

To unload and hoist down 1 unit machine with accessories from low-bed to the ground, uncrate, shift and position into workshop at ground

Machine Spec

1 unit Mazak I200HS

Dim: 3505 x 2170 x 2250 mm

Wt: 11, 130 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

Est:

04/03/23

Time of Operation

10:00 am

Location of Operation

#### 500 Dover Rd – Singapore Polytechnic

W1411, School of Mechanical & Aeronautical Engineering

Equipment Use

1-2 x 633 PALFINGER lorry loader

1 x low-bed

1 x platform - to lay out for machine pathway

1 set rollers/pallet truck

Wire/webbing sling

Hand drill

Canvas/metal plate (for epoxy floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated location

- Hoist down and lay out platform for machine pathway

- To reverse low-bed to near crane

- Hoist up case from low-bed chassis and unload to the ground

- Uncrate case and put aside

- Attached sling to crane hook and secured to rigging point of machine

- Hoist up and detach machine from base

- Swing crane away from base and lower down machine onto platform

- Insert rollers under machine bed

- Transport machine into workshop at ground floor

- Position machine at the designated location

- Engineer to do marking for glow plug

- Shift machine aside

- To drill 18 holes for clamping plates

- Shift back machine to exact location

- Unload machine from roller and insert clamping plates under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani / Kevin Wong

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175