# METHOD OF STATEMENT

Date: 14th Feb 2017

Description of Work

To unload 4 cases of 1 unit LH1210-1600 CMM machine from lorry chassis, uncrate, shift and position into customer premise at ground floor

Machine Spec

Dim: 343 x 213 x 119 cm

Wt: 5860 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

14/02/2017   -   12:00 pm

Location of Operation

#### No 501 Airport Rd

ST Aerospace Engines

Equipment Use

2 x lorry crane

Pallet truck/rollers

Wire/webbing sling

Obstacles

No obstacles on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman, lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lorry crane to park at designated location for the lift

- Hoist up machine from lorry chassis and lower down to the ground

- Uncrate case

- Hoist up and detach machine from base

- Swing machine away from base to loading area

- Insert pallet truck/rollers to machine base

- Shift and position to the designated location at ground floor

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris, will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175