# METHOD OF STATEMENT

Date: 18th Aug 2016

Description of work

To shift out 1 unit Rubber Press machine with accessary from production room to loading area by using forklift, secured machine onto wooden base, hoist up onto lorry chassis and deliver to new location

Machine Spec

1 unit Rubber Press machine

Dim: 325 x 20 x 245 cm

Est wt: 3,500 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

To be advised

Location of Operation

#### 501 Airport Road

ST Aerospace Engines Pte Ltd

Equipment Use

2 x lorry crane

Pallet truck

Wire/webbing sling

1 x 4.5 ton forklift

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (etc.…safety shoes, helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger, signalman and forklift operator

# JOB SCOPE

- Forklift to drive into factory premise

- Fork up machine at machine bed

- Shift out machine from production room to hoisting bay

- Load machine onto wooden base (to secure during transportation)

- Hoist up and load machine onto lorry chassis

- Secured machine with belt prior to moving out of customer premise and transport to new location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175