# METHOD OF STATEMENT

Date: 05th July 2019

Description of Work

To unload 1 unit machine and parts from 1 x 40ft Malaysia lorry and 2 x 40ft low bed, assist engineer to install machine parts, after install, shift and position into premise at ground floor

Machine Spec

Youji CNC Vertical Turning Lathe / VTL2500 ATC – To assemble outside factory

Base & Table Dim: 548 x 415 x 282 cm / Wt: 29, 000 kgs

Column Dim: 530 x 382 x 397 cm / Wt: 30, 000kgs

Conveyor Dim: 621 x 221 x 220 cm / Wt: 2, 200 kgs

1 pallet accessories: 364 x 221 x 220 cm / Wt: 1, 600 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

12-13/07/2019

Time of Operation

09:30 am

Location of Operation

#### 501 Airport Rd

#### ST Aerospace Engines Pte Ltd

Equipment Use

1 x 160 ton mobile crane

1 x 633 lorry loader - PALFINGER PK100002F

1 set wire sling

1 x forklift

Shackle

Rollers/pallet truck

Obstacles

To clear all obstacles on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy mobile crane at designated hoisting location for the lift

- Reverse low bed to near crane

- Hoist up machine/parts from low bed

- Move low bed away

- Lower down machine/part to the ground

- Assist engineer to install machine parts

- After installing, shift and transport machine into inside factory

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine ned

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175