# METHOD OF STATEMENT

Date: 03rd April 2017

Description of work

To shift out and hoist down 1 unit Okuma L270E CNC Lathe machine from 2nd floor to ground floor by using platform and mobile/lorry crane

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

05/04/2017

Location of Operation

#### 505 Yishun Industrial Pk A

PBA (S) Pte Ltd

Equipment Use

1 x 50 ton mobile/lorry crane

Lorry crane

1 x platform

Wire sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris, if any will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, safety harness,  etc…) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Mobile/lorry crane to park at designated location for the hoist

- Hoist up platform to 2nd level opening

- Stabilize platform effectively and ensure it is fully rested on the opening slab

- Use of strong belt to secure front left and right of platform to prevent platform swing outward

- Shift out machine from production and load onto platform

- Secured machine onto platform with strapping belt

- Hoist down platform together with machine to ground floor

- Shift and position machine to designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris, if any will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175