# METHOD OF STATEMENT

Date: 08th Mar 2024

Description of Work

To unload 2 cases from lorry chassis, uncrate, shift and position into customer premise at ground floor

Spec

2 packages of 1 unit Electrical Cabinet

Case 1: 2430 x 1970 x 1670 mm / N, G Wt: 870 kg / 1, 320 kgs

Case 2: 1589 x 830 x 970 mm / N, G Wt: 130 kg / 175 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

12/03/24

Time of Operation

10:30 am

Location of Operation

#### No 51 Calshot Rd

Eagle Services Asia Pte Ltd

Equipment Use

1 x lorry crane

Pallet truck

Webbing sling

Obstacles

No obstacles on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman.

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Hoist down cases from lorry chassis

- Uncrate case and put aside

- Rigger to attached sling to crane hook

- Secured sling to rigging point of load

- Hoist up and detach from base

- Swing load away from base

- Lower down to near ground level

- Insert pallet truck under equipment bed

- Transport into factory premise

- Position at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175