# METHOD OF STATEMENT

Date: 10th Sep 2018

Description of Work

To unload 2 cases from forwarder lorry, uncrate, shift and position into customer premise

Machine Spec

1 unit Measuring Machine

Case 1: 107 x 209 x 140 cm   /   Wt: 2, 994 kgs

Case 2: 107 x 216 x 135 cm   /   Wt: 816 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

11/09/2018   -   09:30 am

Location of Operation

#### No 51 Calshot Rd

Eagle Services Asia Pte Ltd

Equipment Use

1 x lorry crane

Pallet truck

Webbing sling

Obstacles

No obstacles on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, etc..)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman.

# JOB SCOPE

- Lorry crane to park at designated hoisting area

- Unload cases from forwarder lorry

- Uncrate machine case

- Hoist up and detach machine from base

- Swing crane to unloading bay

- Lower down machine to near ground level

- Insert pallet truck to machine stand

- Transport machine into premise

- Shift and position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175