# METHOD OF STATEMENT

Date: 3rd Oct 2018

Description of work

To hoist down 1 unit 110 ton Injection Moulding machine from 4th floor to 3rd floor by using platform and mobile/lorry crane

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

06/10/2018   –   09:40 am

Location of Operation

#### 51 Marsiling Lane

Beyonics International Pte Ltd

Equipment Use

1 x 50 ton mobile crane

1 x 45 ton lorry crane

Pallet truck/rollers

1 x platform

Wire sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile crane operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Mobile crane to park at designated hoisting area for the lift

- Hoist up platform to 4th level opening

- Stabilize platform effectively and ensure it is fully rested on the opening slab

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Shift machine into platform

- Secured machine onto platform with strapping belt

- Hoist down platform together with machine to 3rd floor shutter door opening

- Stabilize platform effectively and ensure it is fully rested on the opening slab

- Shift out machine from platform and position at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175