# METHOD OF STATEMENT

Date: 27th June 2018

Description of work

To unload 1 unit Horizontal Injection machine from forwarder lorry and hoist up to 4th floor by using platform and mobile/lorry crane

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

30/06/2018   –   09:45 am

Location of Operation

#### 51 Marsiling Lane #04-00

Beyonics International Pte Ltd

Equipment Use

1 x 50 ton mobile crane

1 x 45 ton lorry crane

Pallet truck/rollers

1 x platform

Wire sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry/mobile crane operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman.

# JOB SCOPE

- Mobile/lorry crane to park at designated hoisting area for the lift

- Hoist up platform to 4th level opening

- Stabilize platform effectively and ensure it is fully rested on the opening slab

- Tied back platform to a competent point and secured using 45 ton lorry crane

- Use strong belt to secure front left and right of platform to prevent platform swing outward

- 50 ton mobile to hoist up machine to 4th level opening

- Lower down machine onto platform

- Insert rollers to machine base, shift out from platform and position into customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175