# METHOD OF STATEMENT

Date: 24th Jan 2024

Description of work

To shift out 4 unit machine from production area to loading bay, load up onto lorry chassis, transport out lorry from loading bay, hoist up machine and load onto Malaysia lorry

Machine Spec

1 unit Aida NC1-150

1 unit Aida NCI-110

1 unit Yamada Dobby DP25CS

1 unit Yamada Dobby DP-35CS

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

27/01/24

Time of Operation

09:40 am

Location of Operation

#### 51 Marsiling Rd

Beyonics International Pte Ltd

Equipment Use

2-3 x 633 lorry loader

3 x 40ft Malaysia lorry

Rollers

Pallet truck

Hydraulic jack

Woodblock

Wire/webbing sling

Shackle

Obstacles

No obstacle on loading/unloading bay and crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Jack up machine to 3” high

- Insert rollers under machine bed

- To shift out machine from production area to loading bay

- Reverse lorry to loading bay

- Level up lorry chassis with loading bay

- Load up machine onto lorry chassis

- Transport machine out from loading bay

- Deploy lorry at unloading area to transfer machine onto Malaysia lorry

- Rigger will attached sling to crane hook

- Secured sling to rigging point of machine

- Hoist down machine to the ground

- Use 2 x lorry crane to hoist and lay down machine to horizontal position

- Hoist and load up machine onto Malaysia lorry

- Release all sling from machine

- Secured machine with belt prior to moving out customer premise

- Transport and deliver machine to new location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175