# METHOD OF STATEMENT

Date: 17th Aug 2021

Description of work

To unload 1 unit machine from 1 x 20ft open top container, shift and position into customer production area at ground floor

Machine Spec

1 x 80 ton High Speed Press

Dim: 260 x 160 x 285 cm

Wt: 14, 100 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

Est: 02-04/09/21

Time of Operation

09:45 am

Location of Operation

#### 6 Gul St 5

FCI Connectors Pte Ltd

Equipment Use

1 x 50 ton mobile crane

1 x 50 ton lorry crane

Rollers/pallet truck

Wire sling/shackle

1 x forklift

Obstacles

No obstacle on crane parking area and route to machine location

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Setting up of mobile crane at designated location for the lift

- Open up container canvas

- Secure wire sling to machine hoisting point

- Hoist up machine from container and lower down to near ground level

- Insert rollers to machine base and secured with bolts and nuts

- Shift machine into production area

- Position machine at the designated location

- Jack up machine, pull out rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175