# METHOD OF STATEMENT

Date: 06th Nov 2019

Description of work

To hoist down 1 unit CMM machine from 2nd floor and hoist up 1 unit CMM machine to

2nd floor by using platform and lorry crane

Machine Spec

ZEISS CMM (hoist down)

Dim: 1m x 1.2m x 2m

Wt: 1, 000 kg

ALTERA 8.7.6 CMM (hoist up)

Dim: 1.4m x 1.4m x 2.1m

Wt: 1, 250 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

14/11/19   -   09:45 am

Location of Operation

#### 6 Marsiling Lane, Blk C #02-02

GD Precision

Equipment Use

1 x 633 lorry loader

1 x lorry loader

1 x platform

Wire sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoe, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry crane at designated hoisting area for the lift

- Load machine onto platform

- Secured machine onto platform with strapping belt

- Hoist up platform together with machine to 2nd floor door opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the building opening slab

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Insert pallet truck under machine stand

- Shift out machine from platform

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175