# METHOD OF STATEMENT

Date: 15th Jan 2021

Description of work

To hoist down 1 unit CMM machine from 2nd floor by using platform and lorry crane

Machine Spec

1 unit Altera 8.7.6 CMM machine

Dim: 140 x 140 x 210 cm

Wt: 1, 250 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

22/01/2021 (Friday)

Time of Operation

09:45 am

Location of Operation

#### 6 Marsiling Lane, Blk C #02-02

GD Precision

Equipment Use

1 x 633 lorry loader

1 x platform

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoe, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated hoisting area

- Hoist up platform to 2nd floor

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the building opening slab

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Shift out machine from production room and load onto platform

- Secured machine onto platform with strapping belt

- Hoist down platform together with machine and load onto lorry chassis

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175