# METHOD OF STATEMENT

Date: 19th Sep 2017

Description of work

To shift out 3 unit machine from ground floor and transfer to side building, hoist up to 2nd floor by using lorry crane

Machine spec

Maxiem 1530 Waterjet machine

Dim: 4.3m x 2.3m x 2.1m

Wt: 3, 630 kgs

SMS Cutting machine

Dim: 360 x 170

Wt: 3, 700 kgs

1 unit Bending machine

Dim: 240 x 120

Wt: 2, 000 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

21/09/2017   -    10:00 am

Location of Operation

#### No 6 Penjuru Close

#### HOPE Technik Pte Ltd

Equipment Use

1 x 50 ton lorry crane

Rollers/pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lorry crane to park at designated location for the hoist

- Shift out machine and transfer to side building

- Insert wire/webbing sling to machine hoisting point

- Hoist up machine to 2nd floor

- Lower down machine onto building platform

- Insert pallet truck/rollers to machine base

- Shift out machine from platform and position to designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris, if any will be dispose and clear

- Job complete

Remark

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani / Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175