# METHOD OF STATEMENT

Date: 11th Feb 2019

Description of Work

To unload 1 case of 1 unit machine from lorry chassis, uncrate, shift and position to designated location at ground floor

Machine Spec

Global S Green 9.12.08 CMM

Dim: 1477 x 2165 x 3027 mm

Wt: 1, 700 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

18/02/2019   -   09:30 am

Location of Operation

#### 6 Seletar Link

GE Aviation Engine Services

Equipment Use

1 x lorry crane

Pallet truck

Wire/webbing sling

Obstacles

No obstacles on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lorry crane to park at designated location

- Hoist down machine case from lorry chassis

- Uncrate machine case

- Hoist up and detach machine from base

- Swing crane to unloading area

- Lower down machine to near ground level

- Insert pallet truck to machine base

- Shift and transport machine into premise at ground floor

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175