# METHOD OF STATEMENT

Date: 20th July 2017

Description of work

1) To shift out 1 unit used CMM Chameleon machine from customer premise at 2nd floor, load onto platform, secured with strapping belt, hoist down and load onto lorry chassis

2) To unload 1 unit new CMM Global machine from lorry chassis, uncrate, load onto platform, secured with strapping belt, hoist up together with platform to 2nd floor, shift out from platform and position into designated location

Old Machine Spec

1 unit used CMM Chameleon / Model: 09.15.08

Dim: 1750 x 2450 x 2310 mm

Wt: 2600 kgs

New Machine Spec

1 unit CMM Global / Model: 07.10.07

Crate Dim: 3318 x 1818 x 1959 mm

Wt: 2026 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

29/07/2017   -   09:30 am

Location of Operation

#### 62 Loyang Way

#### GE Aviation

Equipment Use

1 x 50 ton lorry crane

1 x platform

Wire/webbing sling

Metal plate/plywood (for floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lorry crane to park at designated location for the hoisting

- Unload case from lorry chassis

- Uncrate machine case

- After uncrate, hoist and detach machine from base

- Load machine onto platform

- Strap and secured machine onto platform with strapping belt

- Hoist up platform together with machine to 2nd level opening

- Stabilize platform effectively and ensure it is fully rested on the opening slab

- Use strong belt to secure front left and right of platform to prevent platform swing outward

- Insert pallet truck to machine base

- Shift out machine from platform and position into customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be dispose and clear

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175