# METHOD OF STATEMENT

Date: 14th June 2016

Description of work

1) To hoist up 15 unit machineries to 3rd floor by using platform and mobile/lorry crane

2) To unload, shift and position 15 unit smaller machine to 3rd floor via cargo lift

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

28-29/06/2016 – 11:00 am

Location of Operation

#### 690 West Camp Road #03-02

(Coway Engineering & Marketing Pte Ltd)

Equipment Use

1 x 50 ton mobile

1 x 50 ton lorry crane

Lorry crane

1 x platform

Wire Sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Assignment of traffic controller to control movement within the vicinity                                                            All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness, etc..)

Lifting machine and lifting gear with M.O.M certificate

# JOB SCOPE

- Mobile/lorry crane to park at designated hoisting area for the lift

- Hoist up platform to 3rd floor opening

- Tied back platform to a competent point and secured using 50 ton lorry crane

- Use strong belt to secured front left and right of platform to prevent platform swing outward

- 50 ton mobile crane to hoist up machine to 3rd floor

- Lower down machine and load onto platform

- Insert pallet truck/rollers to machine base

- Shift out from platform and position into customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175