# METHOD OF STATEMENT

Date: 19th Sep 2018

Description of Work

To unload 2 packages of 1 unit machine from lorry chassis, uncrate, shift and position into customer premise

Machine Spec

1 unit Germany ‘DATRON NEO’ CNC High Speed Milling

Case 1: 148 x 103 x 216 cm

Case 2: 138 x 98 x 132 cm

Total Gross Wt: 1, 115 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

29/09/2018   -   09:30 am

Location of Operation

#### 7 A.M.K St 64

Apple South Asia Pte Ltd

Equipment Use

1 x lorry crane

Pallet truck/rollers

Webbing sling

Obstacles

No obstacles on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lorry crane to park at designated hoisting area for the lift

- Uncrate machine case

- Hoist up and detach machine from base

- Swing crane and lower down machine from lorry chassis to near ground level

- Insert pallet truck to machine base for the move

- Transport machine into premise

- Shift and position machine at the designated location

- Unload machine from pallet truck/roller and let it sit onto machine pad

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175