# METHOD OF STATEMENT

Date: 14th Nov 2018

Description of work

To unload 1 unit Lead Tester from 1 x 40ft OT container and 1 set Full Auto Production Line from 1 x 40ft OT, 1 x 40ft GP and 1 x 20ft GP container, shift and position into customer production area at ground floor

Machine Spec

As per your packing list

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

16/11/2018 – Unload Lead Tester from container, shift and position into production at ground floor

20-21/11/2018 – Unload Full Auto Production Line from container, shift and position into production at ground floor

Location of Operation

#### 7 Benoi Sector

Superior Multi Packaging Limited

Equipment Use

1 x 50 ton lorry crane

1 x lorry crane

Rollers/pallet truck

Wire/webbing sling

1 x forklift

Obstacles

No obstacle on crane parking area and route to machine location

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Setting up of lorry crane at designated location (back to back with container)

- Shift out machine from container

- Load machine onto lorry chassis

- Transport machine to loading area

- Hoist down machine from lorry chassis

- Insert rollers/pallet truck to machine base

- Shift machine into production area and position at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175