# METHOD OF STATEMENT

Date: 06th Sep 2021

Description of work

To shift out 1 unit machine from customer premise to loading area, hoist up and load onto lorry chassis

Machine Spec

1 unit Cut AM500

Dim: 3240 x 1930 x 2530 mm

Wt: 3, 775 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

10/09/21 (Friday)

Time of Operation

09:45 am

Location of Operation

#### 73 Nanyang Drive

Singapore Institute of Mfg Technology

Equipment Use

1 x 633 lorry loader

Pallet truck

Hand jack/crowbar

Wire sling

Canvas/metal sheet (for floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lift up machine to 3” high

- Insert pallet truck under machine bed

- Shift out and transport machine to loading area

- Attached required lifting gears to crane hook and secure to hoisting point of machine to be lifted

- Lorry loader to hoist up machine and load onto lorry chassis

- Secured machine with strapping belt prior to moving out of premise

- Transport and deliver machine to another location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175