# METHOD OF STATEMENT

Date: 26th Nov 2019

Description of work

To shift out equipment to loading area, hoist up onto lorry chassis and transfer to temporary storage area within premise

Equipment Spec

1 unit New Heat Exchanger

Dim: 310 x 80 x 110 cm

Wt: 2, 490 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

28/11/2019   -   09:45 am

Location of Operation

#### No 8 Pandan Ave

Equipment Use

1 x 633 lorry loader

Rollers/pallet truck

1 x forklift

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Shift out equipment from production room to loading area

- Lorry loader to hoist up and load equipment onto lorry chassis

- Secured equipment with strapping belt prior

- Transport and deliver to temporary storage area within premise

- Hoist down equipment from lorry chassis

- Position at temporary designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175