# METHOD OF STATEMENT

Date: 25th Sep 2018

Description of Work

To unload 1 unit machine with accessories from low loader/lorry chassis, shift and position into customer premise at ground floor

Machine Spec

1 unit CTX 2000 TC CNC Lathe

Main Machine: 640 x 290 x 250 cm

Chip Conveyor: 750 x 90 x 170 cm

Chiller: 85 x 50 x 135 cm

Coolant Tank: 190 x 160 x 160 cm

Total Wt: 24, 000 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

28/09/2018   -   11:30 am

Location of Operation

#### 8 Tuas West St

Wilhelm (SEA) Pte Ltd

Equipment Use

1 x 100 lorry crane

1 x lorry crane

1 x low loader

Rollers/pallet truck

Wire/webbing sling

Shackle

Obstacles

No obstacles on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lorry crane to park at designated location for unloading of machine

- Reverse low loader to near crane

- Secured sling onto machine hoisting point

- Hoist up machine form low loader chassis

- Drive low loader away from loading area

- Lower down machine to near ground level

- Swing machine to unloading area

- Insert rollers to machine base for the move

- Transport machine into premise

- Shift and position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris, if any will be clear and dispose

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175