# METHOD OF STATEMENT

Date: 08th Aug 2019

Description of work

1) To hoist up 2 unit forklift to 3rd floor through shutter door opening

2) To shift out and hoist down 11 Order Picker, 2 unit VMA (to dismantle into 3 parts) and 1 lot racking from 3rd floor through shutter door opening and put aside

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

24/08/2019   -   To hoist up 2 unit forklift to 3rd floor

31/08/2019   -   To hoist down Order Picker, VMA and 1 lot racking

Time of Operation

09:30 am

Location of Operation

#### 8 Upper Changi Rd North

Applied Materials

Equipment Use

1 x 100 ton lorry loader

1 set wire sling

1 set webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

1) Access to be restricted to the site area, with pedestrian control to be in force when any potential hazardous movement is to be taken.

2) To barricade work area with safety cones, red/white tape and lifting in operation signage.

3) All work area to be kept clear or trip hazards and a good housekeeping standard is required.

4) All operatives to wear project specific PPE including – safety helmet, safety shoes, safety harness, gloves.

5) A certified lifting supervisor, signaler, riggers to be identified to the lift operator by different colour vest and/or safety helmet.

6) Reversing vehicle to be undertaken with banksman present.

7) All lifting equipment and accessories selected for the operation is to be assessed prior to use for appropriate and up to date maintenance and inspection records, examination and testing certification.

8) Only trained/certified personnel to operate the lifting equipment.

9) Obtain known weight of item to be lifted.

# Work Method for Lifting Operation

Preparation:

1) An assessment will be made on site by the appointed person and/or the crane supervisor as to the weight and suitability of the item to be lifted

2) The current location and the final location of the load to be established

3) Selection, provision and use of suitable crane and lifting equipment is then to be considered in consultation with suitable supplier

The crane and lifting equipment selected for the operation is to be assessed prior to use

All personnel to be issued with a copy of this Method Statement for review

- To barricade off work area

- Shift equipment to near shutter door opening

- Insert sling onto equipment hoisting point

- Lorry crane to hoist up slowly to check on balancing

- When balancing if achieved, hoist down equipment slowly to ground floor

- Unload equipment and put aside

- Release all lifting gears from equipment

- Repeat procedure until all equipment have been hoist down

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175