# METHOD OF STATEMENT

Date: 08th Aug 2019

Description of work

1) To hoist up 2 unit forklift to 3rd floor through shutter door opening

2) To shift out and hoist down 11 Order Picker, 2 unit VMA (to dismantle into 3 parts) and 1 lot racking from 3rd floor through shutter door opening and put aside

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

24/08/2019   -   To hoist up 2 unit forklift to 3rd floor

31/08/2019   -   To hoist down Order Picker, VMA and 1 lot racking

Time of Operation

09:30 am

Location of Operation

#### 8 Upper Changi Rd North

Applied Materials

Equipment Use

1 x 100 ton lorry loader

1 set wire sling

1 set webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated hoisting area for the lift

- To barricade off work area

- Shift equipment to near shutter door opening

- Insert sling onto equipment hoisting point

- Lorry crane to hoist up slowly to check on balancing

- When balancing if achieved, hoist down equipment slowly to ground floor

- Unload equipment and put aside

- Release all lifting gears from equipment

- Repeat procedure until all equipment have been hoist down

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175