# METHOD OF STATEMENT

Date: 03rd July 2019

Description of Work

To unload 5 unit machine from 4 x 40ft HQ container, shift and position into customer premise at ground floor

Machine Spec

SC-25 CNC Lathe

Dim: 224 x 190 x 200 cm

Wt: 9, 000 kgs each

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

11/07/2019

Time of Operation

09:45 am

Location of Operation

#### 80 Tuas West Drive

Dril-Quip Asia Pacific Pte Ltd

Equipment Use

1 x 633 lorry crane

Heavy duty rollers

Pallet truck

Wire sling

1 x forklift

Obstacles

No obstacles on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lorry crane to park at designated location (back to back with container)

- Crane to pull out machine from container onto lorry chassis

- Hoist up and detach machine from bed

- Lower down machine to near ground level

- Swing crane to loading bay

- Insert rollers under machine base

- Shift machine into production at ground floor

- Position machine at the designated location

- Unload machine from rollers and insert leveling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175