# METHOD OF STATEMENT

Date: 06th Nov 2017

Description of work

To hoist up 1 unit SV 110S CNC machine to 2nd floor by using platform and mobile/lorry crane

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

11/11/2017   –   09:30 am

Location of Operation

#### Blk 998 Toa Payoh North #02-25

Micro 21 Precision Industries

Equipment Use

1 x 50 ton mobile/lorry crane

1 x 45 ton lorry crane

Pallet truck/rollers

1 x platform

Wire sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry/mobile crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, safety harness etc..) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Mobile/lorry crane to park at designated hoisting area for the lift

- Hoist up platform to 2nd floor opening

- Stabilize platform effectively and ensure it is fully rested on the opening slab

- Tied back platform to a competent point and secured using 45 ton lorry crane

- Use strong belt to secure front left and right of platform to prevent platform swing outward

- 50 ton mobile/lorry crane to hoist up machine to 2nd floor

- Lower down machine onto platform

- Insert pallet truck/rollers to machine base, shift out from platform and position into customer premise

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris, if any will be clear and dispose

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175