# METHOD OF STATEMENT

Date: 09th July 2018

Description of Work

To dismantle 1 pcs machine door @ weight of 800 kg, load onto pallet and transfer to ground floor via cargo to welding room

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

Est: 11-12/07/2018   -   09:45 am

Location of Operation

#### No 8 Tractor Rd

Qioptiq Singapore Pte Ltd

Equipment Use

1 x lorry crane

Pallet truck

Webbing sling

1 set ‘A’ frame structure with 3 ton chain block

Obstacles

No obstacles on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger and signalman will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, etc..)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job scope

- Using ‘A’ frame structure and chain block to dismantle 1 pcs machine door

- Load onto pallet

- Transport door from 2nd floor to ground floor via cargo lift

- Shift into welding room

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris, if any will be clear and dispose

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175