# METHOD OF STATEMENT

Date: 19th Sep 2017

Description of work

To shift out and hoist down 2 unit CNC machine from 2nd floor, load onto low loader and transport to new premise

Machine Spec

1 unit Mazak E420H-II

Dim: 550 x 345 x 290 cm

Wt: 18, 000 kgs

1 unit DMG DMU 60EVO

Dim: 230 x 300 x 280 cm

Wt: 10, 000 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

TBC

Location of Operation

#### 50 ALPS Ave #02-00

Sandvik Building

Equipment Use

1 x 70 ton mobile crane

1 x 50 ton mobile crane

1 x lorry crane

1 x low loader

1 x platform

Roller/pallet truck

Wire/webbing sling

Metal plate/plywood (for epoxy floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, safety harness,  etc..) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- 2 x mobile crane to park at designated location for the hoist

- Hoist up platform to 2nd level opening

- Stabilize platform effectively and ensure it is fully rested on the opening slab

- Tied back platform to a competent point and secured using 50 ton mobile crane

- Use of strong belt to secure front left and right of platform to prevent platform swing outward

- Shift out machine from production and load onto platform

- 70 ton mobile crane to hoist down machine from 2nd floor

- Load machine onto low loader chassis

- Secured machine with strapping belt prior to moving out of premise and transport to new location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris, if any will be clear and dispose

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175