# METHOD OF STATEMENT

Date: 07th Aug 2017

Description of work

To shift out and hoist down 4 unit I Cut 380Ti machine with accessories from 2nd floor and load onto 1 x 40ft and 1 x 20ft M’sia lorry

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

Est: 14/08/2015   -   10:00am

Location of Operation

#### No 6 Marsiling Lane, 2nd floor

Equipment Use

1 x 50 ton mobile crane

1 x 50 ton lorry crane

1 x platform

Rollers / pallet truck

Wire sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, safety harness etc..) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# Job Scope

- Mobile/lorry crane to park at designated hoisting area

- Hoist up platform to 2nd level opening

- Tied back platform to competent point and secured using 50 ton lorry crane

- Stabilize platform and ensure it is fully rested on the opening slab

- Use strong belt to secure front left and right of platform to prevent platform swing outward

- Shift out machine from production into platform

- Hoist down machine and load onto M’sia lorry

- Secured machine with strapping belt prior to moving out of premise

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris, if any will be clear and dispose

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175