# METHOD OF STATEMENT

Date: 26th May 2017

Description of work

1) To shift out 1 unit Kingang Furnace machine from shelter and transfer to welding area

2) To shift out 1 unit Longport Oven from Bay 6, hoist up onto lorry chassis and transfer to welding area

3) Using lorry crane to relocate 2 unit Manu Pillar within old welding area

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

30/05/17   -   10:00 am

Location of Operation

#### No 273 Jalan Ahmad Ibrahim

(Baker Hughes Singapore)

Equipment Use

1 x 35 ton lorry crane

Crowbar/manual jack

Rollers/pallet truck

Wire/webbing sling

Obstacles

Working area to be keep clear of any obstruction

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, etc..)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lift up machine to 3” high using crowbar/manual Jack

- Insert pallet truck/rollers to machine base

- Shift out to loading area

- Hoist up and load onto lorry chassis

- Lorry crane to transfer to new location

- Unload, shift and position machine at new location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris, if any will be clear and dispose

- Job complete

Remark

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175