# METHOD OF STATEMENT

Date: 23rd Nov 2021

Description of work

To hoist up 1 unit machine to 2nd floor through gate opening by using platform and mobile/lorry crane

# Machine moving plan

- Deployment of mobile crane at designated hoisting area

- Lorry to transport cases into site premise

- Hoist down cases from lorry chassis to the ground

- To uncrate machine case

- Attached required lifting gears to crane hook and secured to rigging point of machine to be lifted

- Hoist up and detach machine from base

- Load machine onto platform

- Secured machine onto platform with strapping belt

- Mobile crane to hoist up platform together with machine to 2nd floor gate opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the building opening slab

- Use strong belt to secure one side of platform onto building pillar and another side onto side rail handle to prevent platform swing outward

- Workers to put on safety harness before stepping into the work platform

- Insert pallet truck under machine base

- Move out machine from platform

- To lay out canvas/metal plate onto flooring for floor protection

- Shift machine into CMM room

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175