# METHOD OF STATEMENT

Date: 23rd Nov 2021

Description of work

To hoist up 1 unit machine to 2nd floor through swing gate opening by using platform and mobile/lorry crane

Machine Spec

CMM Global S Blue 12.22.10

Dim: 1898 x 3605 x 3488 mm

Wt: 5, 750 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

06/12/21 (Mon)

Time of Operation

09:40 am

Duration of Operation

09:40 am   –   12:00 pm

Location of Operation

#### 10 Calshot Rd

#### SAESL

Equipment Use

1 x 50 ton mobile crane

2 x lorry loader

1 x platform

Wire sling

Webbing sling

Shackle

Rollers/pallet truck

Canvas/metal plate (to lay out for floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crew certificate)

P.P.E Required

Safety Shoe

Safety Helmet

Safety Harness

Self Retracting Lifeline

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope / Moving Plan

- Deployment of mobile crane at designated hoisting area

- Lorry to transport cases into site premise

- Hoist down cases from lorry chassis to the ground

- To uncrate machine case

- Attached required lifting gears to crane hook and secured to rigging point of machine to be lifted

- Hoist up and detach machine from base

- Load machine onto platform

- Secured machine onto platform with strapping belt

- Mobile crane to hoist up platform together with machine to 2nd floor gate opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the building opening slab

- Use strong belt to secure one side of platform onto building pillar and another side onto side rail handle to prevent platform swing outward

- Workers to put on safety harness before stepping into the work platform

- Insert pallet truck under machine base

- Move out machine from platform

- To lay out canvas/metal plate onto flooring for floor protection

- Shift machine into CMM room

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175