# METHOD OF STATEMENT

Date: 20th June 2022

Description of work

1) To hoist up 3 unit machine to 4th floor through door opening by using platform and mobile/lorry crane

2) To hoist down 1 unit machine from 4th floor to the ground, load onto lorry chassis and deliver to Eric Wong warehouse for packing

Machine Spec

Hoist up

2 unit Form P350 machine

Dim: 215 x 169 x 220 cm

Wt: 2, 400 kg

1 unit WPI Work Partner

Dim: 203 x 129 x 237 cm

Wt: 1, 100 kg

Hoist down

1 unit Roboform

Dim: 255 x 175 x 225 cm

Wt: 2, 400 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

25/06/22 (Saturday)

Time of Operation

09:45 am

Location of Operation

#### 11 International Business Pk

Hi-P International Ltd

Equipment Use

1 x 50 ton mobile crane

2 x lorry loader

1 x forklift

1 x platform

Wire/webbing sling

Shackle

Rollers/pallet truck

Obstacles

To clear all obstacle around crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crew certificate)

P.P.E Required

Safety Shoe

Safety Helmet

Safety Harness

Self Retracting Lifeline

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M qualified lifting supervisor, rigger and signalman

# Job Scope

- Deployment of mobile/lorry crane at designated hoisting area

- Lorry to transport machine into site premise

- Using forklift to fork down machine from lorry chassis

- Uncrate case

- Fork up and detach machine from base

- Load machine onto platform

- Secured machine onto platform with strapping belt

- Mobile crane to hoist up platform together with machine to 3rd floor gate opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the building opening slab

- Tied back platform to competent point and secured using 1 x lorry crane

- Use strong belt to secure left and right side of platform onto building railing/pillar to prevent platform swing outward

- Workers to put on safety harness before stepping into the work platform

- Insert pallet truck under machine base

- Move out machine from platform

- Transport machine into building premise

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani / Kevin Wong

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175