# METHOD OF STATEMENT

Date: 23rd Apr 2022

Description of work

To hoist up equipment to 5th floor through shutter door opening by using platform and mobile/lorry crane

Equipment Spec

1 pallet of 1 unit storage panel

Dim: 290 x 100 x 100 cm

Wt: 950 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

23/04/22

Time of Operation

09:45 am

Location of Operation

#### 2 A.M.K Drive

#### ITE College Central

Equipment Use

1 x 50 ton mobile crane

1-2 x lorry loader

1 x platform

Wire/webbing sling

Shackle

Rollers/pallet truck

Obstacles

No obstacle on crane parking area and route to equipment positioning

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crew certificate)

P.P.E Required

Safety Shoe

Safety Helmet

Safety Harness

Self Retracting Lifeline

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M qualified lifting supervisor, rigger and signalman

# Job Scope

- Deployment of mobile crane at designated hoisting area

- Secure equipment onto platform with strapping belt

- To hoist up platform together with equipment to 5th floor shutter gate opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the building opening slab

- Use strong belt to secure left and right side of platform onto building railing/pillar to prevent platform swing outward

- Workers to put on safety harness before stepping into the work platform

- Insert pallet truck under equipment base

- Move out equipment from platform

- Transport equipment into building premise

- Position equipment at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani / Kevin Wong

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175