# METHOD OF STATEMENT

Date: 24th Apr 2024

Description of work

To hoist up 1 unit machine to 5th floor through swing gate opening by using platform and mobile/lorry crane

Machine Spec

1 x NXV 600

Dim: 290 x 185 x 228 cm

Wt: 3, 500 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

30/04/24 (Tue)

Time of Operation

9:45 am

Location of Operation

#### Blk 20 Woodlands Link #05-27

#### Skillmax Precision Technologies (S) Pte Ltd

Equipment Use

1 x 50 ton mobile crane

2 x lorry crane

1 x platform

Wire sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy mobile/lorry crane at designated hoisting area for the lift

- Load up machine onto platform

- Secured machine onto platform with strapping belt

- Rigger to insert sling to crane hook

- Secured sling to rigging point of platform

- Hoist up platform together with machine to 5th floor swing gate opening

- Stabilize platform effectively and ensure platform 3ft metal protruding is fully lean and rested on the swing gate opening slab

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Insert pallet truck/rollers under machine base

- Use of machine pulley to slowly release and control moving speed of machine

- Shift out machine from platform

- Transport machine into customer premise

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175