# METHOD OF STATEMENT

Date: 20th June 2024

Description of work

To hoist up 3 unit item of 1 set machine to 2nd floor through swing gate opening by using platform and lorry crane

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

29/06/24 (Sat)

Time of Operation

Estimate: 13:00 pm

Location of Operation

#### Blk 30 Kallang Place #02-21

#### Energy Press Pte Ltd

Equipment Use

1 x 50 ton lorry crane

1 x lorry crane

1 x platform

Wire sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry crane at designated hoisting area for the lift

- Load up machine onto platform

- Secured machine onto platform with strapping belt

- Rigger to insert sling to crane hook

- Secured sling to rigging point of platform

- Hoist up platform together with machine to 2nd floor swing gate opening

- Stabilize platform effectively and ensure platform 3ft metal protruding is fully lean and rested on the swing gate opening slab

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Insert pallet truck/rollers under machine base

- Shift out machine from platform

- Transport machine into customer premise

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175